RELIANCE CONSUMER PRODUCTS LIMITED (RCPL)

Campa Cola Manufacturing Division

Track & Trace Digital Application Suite

Software Design Requirements & Screen Specification

| Document Reference | RCPL-TT-BRD-003 |
| --- | --- |
| Version | 3.1 — Customer Design Review Edition (Full Scope) |
| Date | June 2026 |
| Prepared By | Carbynetech Technologies Pvt. Ltd., Hyderabad |
| Client | Reliance Consumer Products Limited (RCPL) |
| Classification | CONFIDENTIAL — Restricted Distribution |

Automation Partner: Stellar Automation  |  MES Software: Carbynetech Technologies

# Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | May 2026 | Carbynetech | Initial BRD — scope and module outline |
| 2.0 | June 2026 | Carbynetech | Elaborated functional requirements, user stories, business flows |
| 3.1 | June 2026 | Carbynetech | Added Sections 4.8–4.12: Carton CW Reject & Mandatory QA Rework, Case Closure & Serial Print, Palletising, QA Pallet Inspection, FG Warehouse Receipt. Updated Exception Engine, Traceability, Roles, and Open Items accordingly. |

# How to Use This Document

This document is organised as a design review reference. For each major process step the document provides:

- A step-by-step workflow table describing who does what and how the system responds

- A Screen Specification table listing every field on the operator screen — label, input type, validation rules, and whether it is mandatory

- Exception handling rules for that process area

- Amber boxes flagging open questions that need customer confirmation before development begins

- Teal recommendation boxes suggesting best-practice design choices

During the design review session, RCPL's factory team should validate each Screen Specification table row by row, confirming or adjusting field names, validation rules, and mandatory flags to match actual plant operations.

# 1. Executive Summary

Reliance Consumer Products Limited (RCPL), operating under the Campa Cola brand, has commissioned a Track & Trace Digital Application Suite (referred to as DIGITAL throughout this document) for its beverage manufacturing plant. The system integrates with the Siemens/Stellar automation layer to provide comprehensive visibility, control, and traceability across the entire production value chain — from inbound raw material receipt through finished goods dispatch.

## 1.1 Business Problem Statement

RCPL's Campa Cola manufacturing operations greenfield plant, would require unified digital layer to capture, validate, and trace production events. Key gaps include:

- Digital linkage between raw material vendor lots and finished goods cartons

- Eliminate Manual, paper-based GRN and quality inspection workflows causing delays and errors

- Real-time exception detection at dumping and packing stations

- Ability to limit product recalls to only affected batches

- Shop floor visibility into production orders

- Material balance reconciliation at primary packing (giveaway, check weigher rejects) and secondary packing to know the yield loss

## 1.2 Key Business Benefits

| Benefit Area | Expected Outcome |
| --- | --- |
| Traceability | Forward and backward trace from vendor lot to carton in < 30 seconds |
| Quality Control | 100% digital QC disposition — no uninspected material reaches production |
| Inventory Accuracy | Real-time RM, WIP and FG inventory with FEFO compliance enforcement |
| Material Reconciliation | Primary packing material balance: bags loaded vs. pouches produced vs. giveaway vs. rejects |
| Exception Response | Automated exception alerts with role-based approval — eliminating verbal overrides |
| Compliance | Full operator-level audit trail for every transaction — FSSAI and customer audit ready |

# 2. Scope of Work

## 2.1 In-Scope Software Deliverables

| # | Deliverable | Description |
| --- | --- | --- |
| 1 | Event Data Integration | Read-only ingestion from Stellar SQL Server event tables via scheduled polling or CDC |
| 2 | Master Data Portal | Materials, Recipes/BOM, Assets, Tags, Tray Definitions, Users, Tolerances, Rejection Codes |
| 3 | Station Operator Screens (7) | RM Receiving, Dumping Material Request, Dumping Scan, Primary Packing, ISA/Intermediate Storage, Secondary Packing, QA |
| 4 | Android Mobile Apps (4) | GRN & Label, Dumping Scan, Tray & Pouch, Carton Scan |
| 5 | Exception & Workflow Engine | Auto-detection, exception queue, role-based approvals, escalation, audit log |
| 6 | Traceability Module | Genealogy engine: Vendor Lot → Batch → Pouch → Tray → Carton; forward & backward search |
| 7 | Plant-Wide Dashboard | Real-time production, inventory, machine performance, exception KPIs |
| 8 | Reports | Batch History, Material Balance, Rejection Analysis, Traceability, Production, Inventory |
| 9 | RBAC | Role-based access with 7 defined roles; PIN-based supervisor override |
| 10 | Label Printing | Zebra/SATO printer integration for GRN, pallet, pouch, carton QR labels |
| 11 | Rework Order Management | Check weigher rejects tracked as rework orders linked to parent job order |

## 2.2 Explicitly Out of Scope

| Out-of-Scope Item | Disposition |
| --- | --- |
| SAP MM/PP/WM Integration | Future Phase — separate Change Request required |
| Packaging Materials (PM) Tracking | PM Materials not in scope — separate change request required |
| PLC/SCADA Write-Back | DIGITAL is read-only from OT layer; no machine control from application |
| Kepware OPC Integration | Stellar delivers MQTT→SQL pipeline; Kepware not required |
| LIMS — Lab Test Parameters | QA test parameters out of scope; pass/fail disposition only captured |
| Hardware Procurement | Servers, IPCs, Android devices, printers — customer/Stellar responsibility |

# 3. Production Process Flows and Screen Specifications

This section is the primary design review reference. It covers the complete production value chain in a single consolidated section to avoid process step duplication. Each sub-section contains the detailed workflow, the screen-by-screen field specification, exception handling rules, and open design questions.

Overall Production Value Chain

Stage 1: Raw Material Receiving & QC Inspection

Stage 2: Dumping Material Requisition (RM Warehouse → Shopfloor)

Stage 3: Dumping / Batching (Bag Scan → Batch Creation)

Stage 4: Primary Packing (Rovema Machine — Pouching)

Stage 5: Check Weigher Rework & Material Balance Reconciliation

Stage 6: Intermediate Storage Area (ISA — Tray Management)

Stage 7: Secondary Packing (Cartoning — Manual )

Stage 8: Finished Goods Posting & Dispatch

## 3.1 Raw Material Receiving & Quality Inspection

This stage covers vendor truck arrival through material clearance and put away. The Store Operator performs GRN, the QA Supervisor records inspection results, and the system enforces a hard block preventing any material from being issued to production until QC disposition is complete.

### 3.1.1 Step-by-Step Workflow

| Step # | Actor | Action Performed | System Response / Outcome |
| --- | --- | --- | --- |
| 1 | Gate Guard | Records truck arrival number at gate | System logs truck arrival event with timestamp |
| 2 | Store Operator | Opens 'New GRN' screen on IPC or Android App | System displays PO selection screen with list of open POs |
| 3 | Store Operator | Selects or searches Purchase Order / Transfer Order | System auto-populates: Vendor Name, Material List, Expected Quantities |
| 4 | Store Operator | Scans vendor barcode or manually enters material details | If GS1 barcode: auto-populate Vendor Lot, Material Code, Qty, Expiry. If manual: all fields editable. |
| 5 | Store Operator | Enters: Vendor Lot No., DC No., Received Qty, Pallet/Bag Count, Vehicle No. | System validates mandatory fields. Checks quantity vs PO (tolerance configurable ±3%). If variance > tolerance, GRN flagged for approval. |
| 6 | Store Operator | Submits GRN | System generates MES Internal Lot ID (format: LOT-YYYYMMDD-NNN). Sets material status = UNDER_INSPECTION. Auto-triggers IQC request. Prints GRN label. |
| 7 | Store Operator | Prints pallet labels and places material in quarantine zone | Label includes: Lot ID, Material, Qty, Vendor, Expiry, QC: PENDING (barcode/QR). Dock-to-store timer starts. |
| 8 | QA Supervisor | Receives pending inspection alert on QA station screen | Alert shows: Material, Vendor, Lot ID, Qty, GRN time, Waiting duration. QC pending timer visible. |
| 9 | QA Supervisor | Records inspection decision: PASS / HOLD / REJECT | PASS: material → APPROVED, green label printed. HOLD: material blocked, yellow label, re-inspection date set. REJECT: inventory zeroed, red label, return-to-vendor initiated. |
| 10 | Store Operator | Scans Aisle-Bay location label and confirms put away. No of pallets and no of bags on that pallet, needs to enter and that info on barcode for trace purpose. Pallet sticker would have RM SKU, lot Id, Pallet Id & Bags count. Mfg date. | System records Lot ID → Location mapping. Inventory updated. Material available for production issuance (if APPROVED). |

### 3.1.2 Exception Handling — RM Receiving

| Exception Code | Trigger Condition | System Action | Required Role | Resolution Options |
| --- | --- | --- | --- | --- |
| EX-GRN-01 | Received qty variance > configured tolerance (±3% of PO qty) | GRN placed on hold. Store Manager notified. Submission blocked. | Store Manager | Approve Variance ¦ Reject and Return to Vendor |
| EX-GRN-02 | Material code not found in Material Master | GRN submission blocked. Error shown: 'Material not configured — contact Admin'. | Admin | Add Material to Master and retry |
| EX-QC-01 | QC pending > 4 hours (configurable) | Auto-escalation alert sent to Production Manager. Timer badge turns red on dashboard. | Production Manager | Acknowledge ¦ Reassign Inspector |
| EX-QC-02 | Lot REJECTED — material in quarantine zone | Store Manager alerted. Return-to-vendor workflow initiated. Inventory zeroed. | Store Manager | Confirm Return ¦ Raise Credit Note |

| OQ-01 | OPEN QUESTION: Please confirm the list of IQC rejection reason codes to be configured in the system. Carbynetech will set these up in master data before UAT. |
| --- | --- |

## 3.2 Dumping Material Requisition — RM Warehouse to Shopfloor

This is a new workflow formalising the digital communication between the dumping floor operator and the RM warehouse store operator. The dumping floor is one level above the RM warehouse. All material movement requests are sent digitally — eliminating verbal or paper-based requests.

### 3.2.1 Step-by-Step Workflow

| Step # | Actor | Action Performed | System Response / Outcome |
| --- | --- | --- | --- |
| 1 | Production Planner | Creates the order for that shift /day Dumping order batch size should be based on corresponding RM bag size. Operational loss for component around 2% which needs to include in order calculation. | System calculates required BOM & availability. Assigning of machines over which PO carried out with priority |
| 2 | Production Manager | Review the created Production Order and approves along with priority/sequence. | System will calculate RM required to execute the approved PO. |
| 3 | Planning operator/ Store Operator | Reviews shift/day production orders on the Dumping IPC screen and determines RM quantities required. Rounding off RM quantities. Partial close of order, if change in plan or machine discontinued | System calculates RM net requirements from the BOM for all released orders in the shift queue. Displays: Material, Required Qty, Available in staging, Shortfall. Orders are created for machine (3 machines available) |
| 4 | Dumping Operator/Supervisor | Raises digital Material Requisition for the shift requirements. | Requisition includes: Order Reference, Material List, Required Qty per material, No of pallets |
| 5 | Store Operator | Reviews requisition on Store IPC screen and validates against available approved stock | Requisition request sent digitally to the Store Operator's IPC screen & Dumping operator. System shows requisition with suggested lots for each material (FEFO order). Store Operator verifies stock availability. Approves requisition or flags shortfall. |
| 6 | Store Operator | Assigns pick tasks to the Store pick operator | Pick task appears on the Store Operator's screen (or handheld device): Material, Lot ID, Qty, Source Storage/Aisle-Bay location, Destination (Staging Location 1/2/3). |
| 7 | Store operator | Operator will print & paste labels for each bag, based on pallets to be picked. Picks pallets from Storage location and lifts to Machine dumping area (one floor up) | System displays pick list on: Pallet/Lot ID, Storage Location, Qty to pick. Bags are rounded up to whole pallet/order size as per batch rounding logic. |
| 8 | Store operator | Operator would scan the staging location barcode. Confirm placing of material at the Machine staging location (one of 3 positions near primary pack machines).Inventory quantities updates needs to reflected. | Operator scans machine staging location, pallet ID, bag id at the Machine staging location. System records: Lot ID, Staging Location (1/2/3), Pallet Id, Bag Id, Operator name, timestamp, Quantity received at staging. Staging inventory updated. |
| 9 | System | Staging location inventory updated | Dumping IPC screen shows material as 'Available at Staging' with location. Dumping operator can now begin bag scanning against production order. |

## 3.3 Dumping & Batching — Bag Scan and Batch Creation

This stage covers the actual mixing/batching process. The Dumping Operator scans RM bags from the staging location into the production order BOM. Each scan is validated in real time. On completion, a Batch ID is generated, and consumption is posted to inventory.

### 3.3.1 Step-by-Step Workflow

| Step # | Actor | Action Performed | System Response / Outcome |
| --- | --- | --- | --- |
| 1 | Rovema Machine Operator | Operator would see the released production order and enters the batchNo Rovema Machine for pouch serial number generation | BatchNo is same as orderId. Naming Convention can be suggested. |
| 2 | System | PLC would signal for Machine Start and it is updated as a production order start. | Machine State Signal should be displayed on the production Order. Order Status is updated accordingly. |
| 3 | Dumping Operator | Scans each RM bag label from the staging location. | System validates in < 2 seconds: (a) Lot ID exists, (b) QC status = APPROVED, (c) Material matches BOM line, (d) Lot not expired, (e) Not consumed in another active batch. Successful scan: bag count incremented, visual progress bar updated. |
| 4 | System | Displays running totals per BOM line | Progress bar per material: Green = within tolerance, Amber = approaching limit, Red = exceeded. Bag count shows X of N bags required. |
| 5 | Dumping Operator | Continues scanning until all bags as per the BOM loaded | 'Confirm Dump Complete' button activates when all lines are within tolerance. If excess quantity scanned, Supervisor override required. |
| 6 | Dumping Operator | Confirms Dump Complete | System generates Batch ID (BATCH-YYYYMMDD-SEQ). All scanned Lot IDs linked to Batch ID. Consumption posted to inventory. Batch status → READY_FOR_PACKING. |
| 7 | System | When material from different lots or suppliers is received and used for the same production order, pouch-to-vendor lot traceability can be maintained using the bag scanning timestamp recorded at the time of material dumping. |  |
| 8 | Dumping Operator | Enters leftover/partial bag quantities for each partially-consumed lot. | System prompts: 'Enter leftover qty for Lot XXXX'. Leftover posted to inventory with status = PARTIAL_USED. If leftover > configurable threshold (e.g. 30% of bag), QA re-inspection flag set. |

### 3.3.2 Exception Handling — Dumping

| Exception Code | Trigger Condition | System Action | Required Role | Resolution Options |
| --- | --- | --- | --- | --- |
| EX-DMP-01 | Scanned bag material does not match any BOM line | Scan Rejected. | Production Supervisor | Notification has to go to the shift supervisor. |
| EX-DMP-02 | Scanned lot has QC status ≠ APPROVED | Scan rejected. Error: 'Lot [ID] status is [HOLD/UNDER_INSPECTION] — cannot use'. Station halted. | Production Supervisor | Override with Justification ¦ Source replacement lot |
| EX-DMP-03 | Cumulative scanned qty > BOM required qty + tolerance | Warning alert. Progress bar turns red. Supervisor override required before next scan. | Production Supervisor | Approve Excess ¦ Adjust Batch Size |
| EX-DMP-04 | Lot is expired (past shelf life date) | Scan rejected immediately. Error shown with expiry date. | QA Supervisor | Confirm Disposal ¦ Request Return to Vendor |
| EX-DMP-05 | Lot already consumed in another active batch | Duplicate consumption block. Error: 'Lot already linked to Batch [ID]'. | Production Supervisor | Investigate ¦ Override if physically separate split lot |

## 3.4 Primary Packing — Rovema Machine Operations

The Primary Packing section receives the assembled batch from the Dumping stage and drives the Rovema pouch filling machine. Rovema PLC sends live machine events to Stellar SQL, validates each pouch serial, tracks check weigher pass/fail, and manages tray assignment for downstream traceability.

### 3.4.1 Step-by-Step Workflow

| Step # | Actor | Action Performed | System Response / Outcome |
| --- | --- | --- | --- |
| 1 | System | Batch from Dumping becomes available on Primary Packing screen | Batch ID, Product SKU, Recipe, Batch Size, and linked RM Lot IDs displayed on Packing IPC screen. Operator confirms batch start. |
| 2 | System | Rovema machine produces pouches — each pouch serial received from machine | Each serial validated: (a) no duplicate in current batch, (b) no duplicate in historical batches. If duplicate detected: reject gate opened, exception raised. |
| 3 | Primary Pack Operator | Confirms machine start and active batch | Machine status read from the PLC Tag.Live pouch counter begins. Order status → IN_PACKING. |
| 4 | System | Check weigher result received per pouch | PASS: pouch assigned to active tray. FAIL: pouch diverted to reject gate. Reject count incremented. If fail rate > 2% in last 100 pouches: amber alert raised. |
| 5 | System | Scans Barcode on the tray to begin tray assignment. | System validates tray: registered in Tray Master, status = EMPTY. Tray linked to current Batch. Subsequent check-weigher PASS pouches assigned to this tray. |
| 6 | System | Tray fills to capacity | System (PLC Communicates to HMI) shows X of N pouches in current tray. When N reached: 'TRAY FULL' banner. Operator confirms tray closure. Tray status → FULL. Prompts next tray scan. |
| 7 | Primary Pack Operator | Closes batch on order completion | Batch Closure Summary generated: Total pouches, Total rejects, Yield %. If yield < threshold (configurable, e.g. 95%), manager approval required before closure. |
| 8 | System | PLC pushes the data to Unified Scada in real time and track and trace reads the data from the Unified Scada. | TrayId,pouchId, pouch Info |

### 3.4.2 Exception Handling — Primary Packing

| Exception Code | Trigger Condition | System Action | Required Role | Resolution Options |
| --- | --- | --- | --- | --- |
| EX-PPK-01 | Duplicate pouch serial detected (within batch or historical) | Duplicate pouch detection would be done by the track and trace and notified. | Operator | Investigate ¦ Confirm Scrapped |
| EX-PPK-02 | Check weigher fail rate > configured threshold in last 10 pouches. | Amber alert on screen and dashboard. Operator notified automatically. | Operator | Halt Line ¦ Adjust CW Settings (Stellar) |
| EX-PPK-03 | Machine status = FAULT received from PLC | Red fault banner on packing screen. Duration tracked. Alert sent to Production Manager. | Production Manager | Acknowledge ¦ Resume after Stellar fix |
| EX-PPK-04 | Yield % at closure < configured threshold | Batch closure will be blocked until it is approved by the manager. The manager must provide a justification before granting approval. | Production Manager | Approve with Justification ¦ Reject and Investigate |
| EX-PPK-05 | tray scanned but status ≠ EMPTY | Scan rejected. Error: 'Tray [ID] already in use / status = [current status]'. | Primary Pack Operator | Scan correct empty tray |

## 3.5 Check Weigher Rework Management & Primary Packing Material Balance

Check weigher rejects from the primary packing line are collected in a rework bin at the dumping floor. These rejects are NOT discarded automatically — they must be QA-inspected within the shift for moisture/usability and either reworked (re-packed) or accounted for as confirmed rejects. This section defines the rework order workflow and the material balance reconciliation that closes out the primary packing job order.

### 3.5.1 Check Weigher Rework Workflow

| Step # | Actor | Action Performed | System Response / Outcome |
| --- | --- | --- | --- |
| 1 | Primary Pack Operator | Check-weigher failed pouches are physically collected in the rework bin near the machine like underweight, overweight, torn pouch | DIGITAL records each failed pouch serial with fail reason (underweight/overweight) and timestamp. Running count shown on packing screen. |
| 2 | Primary Pack Operator | Rework Inspection request should be created and that need to be communicated with the QA Supervisor. Each rework bin should have rejected pouches belongs to the same SKU, no mix up should happen. |  |
| 3 | QA Supervisor | Inspects rework bin and checks for moisture and usability and gives an approval for the rework order. | QA opens Rework Inspection screen. Scans or enters rework pouch. Records weight of rework batch. Records QA decision: REWORKABLE or REJECT (dispose). |
|  | Primary Pack Operator | Open the pouches and cumulate entire rework in a bin. |  |
| 4 | System | Rework Order created — linked to parent job order. | System creates a Rework Order linked to the original primary packing Job Order ID and Batch ID. Rework Order ID = RWK-[parent-job-order]-NNN. This ensures material reconciliation is closed under the same production order. |
| 5 | Primary Pack Operator | Reworkable pouches are re-introduced into the line within the same shift or at end of recipe | Rework pouches scanned back into the packing line under the Rework Order. Each rework pouch assigned a new pouch serial (flagged as REWORK origin). Linked to parent batch genealogy. |
| 6 | System | Material balance reconciliation calculated at batch closure | System computes: Bags Loaded (kg) vs. Pouches Produced (kg) vs. Giveaway from CW (kg) vs. Confirmed Rejects (kg). Balance = 0 within tolerance. Variance captured with reason codes. |
| 7 | Production Manager | Reviews material balance report at batch closure | Material Balance summary shown on batch closure screen. Significant variance requires manager sign-off with reason. |

## 3.6 Intermediate Storage Area (ISA) — Tray Management

The Intermediate Storage Area (ISA) is a buffer zone between Primary Packing and Secondary Packing. All trays from primary packing are moved to the ISA. Trays are placed on pallets and positioned in aisles and bays. RFID and barcode scanning are used to record pallet and tray positions. This section defines the ISA management screens that were missing from the previous document version.

### 3.6.1 Step-by-Step Workflow

| Step # | Actor | Action Performed | System Response / Outcome |
| --- | --- | --- | --- |
| 1 | Primary Pack Operator | Closes a full tray and transfers it to the ISA Pallet. | Tray status updated to FULL. Tray ID and Batch linkage recorded. Physical tray moved to ISA trolley in aisle. |
| 2 | RFID Auto Scanner | Scans tray RFID tag on arrival at ISA. | Tray details retrieved: Tray ID, Product, Batch ID, Pouch Count. System confirms tray received at ISA. |
| 3 | System | ISA inventory maintained | Live view shows all trays in ISA: Tray ID, Product, Batch, Pouch Count, Location, Age, Status. |
| 4 | Secondary Packing (Planner/Operator) | Requests tray issuance for a secondary packing order | System suggests trays for issuance based on FEFO and product match. Operator confirms tray selection. |
| 5 | ISA Operator | Scans tray RFID to confirm issuance to Secondary Packing | Tray status → ISSUED. Inventory decremented. Secondary Packing screen shows tray as available. |

## 3.7 Secondary Packing — Carton Operations

Secondary packing assembles finished goods cartons from pouches held in ISA trays. Three to five operators stand at the carton filling station, each assigned to a specific recipe pouch type. Each operator picks their assigned pouch type and places it into the carton. A display screen at each operator position guides the picking. An RFID/barcode reader at each position confirms the pick.

### 3.7.1 Operator Station Design

For a secondary packing recipe requiring multiple pouch variants, each person is assigned to pick one specific recipe variant. Example: Recipe with 4 pouch types (A, B, C, D) → 4 operators, each assigned one type. The carton moves along the station. Each operator sees on their screen:

- Which pouch type to pick (large clear display)

- Pick status — whether their pick has been scanned and confirmed by the reader

- Carton association — which carton ID this pick is contributing to

- Running count — how many cartons completed so far this shift

### 3.7.2 Carton ID and PLC Memory Handling

Important design note: The carton QR label is printed AFTER the final check weigher, not at the filling station. However, the carton information (pouch genealogy) must be tracked from the moment filling starts. The design approach is:

- PLC assigns a temporary carton tracking sequence number at the start of filling

- This sequence number is held in PLC memory throughout filling and check weigher passage

- Stellar writes the sequence number and all associated pouch serial events to SQL

- Track and trace reads these events and maintains the pre-label carton genealogy in staging memory

- On check weigher PASS: DIGITAL generates the permanent Carton ID, prints the QR label, and posts to FG inventory

### 3.7.3 Step-by-Step Workflow

| Step # | Actor | Action Performed | System Response / Outcome |
| --- | --- | --- | --- |
| 1 | Production Planner | Secondary packing production order to be created for that FG line, Quantity and Shift/Day. |  |
| 2 | Production Manager | Production manager has to approve the orders. |  |
| 3 | Secondary Pack Supervisor | Material Requestion slip would be sent to the ISA Operator by Order Wise/ WorkStation. |  |
| 4 | ISA Operator | ISA Operator will acknowledge the request and issues trays to the Secondary Packing respective workstation. |  |
| 5 | Rovema Label Printer. | Has to do the manual label setting for the SKU to be configured. |  |
| 6 | Secondary Pack Supervisor | Order to be started manually for the line. |  |
| 7 | System | Operator screens at each position is activated. |  |
| 8 | Each Operator (Positions 1-N) | Sees assigned pouch type on their display screen. | Display shows: Pouch Type, Product Name, SKU image (if available), Pick Count this shift. Reader at each position monitors picks. |
| 9 | System | PLC would give the carton Id tag | DIGITAL records t1 timestamp. Temporary carton sequence begins. All pouch scans from t1 onward attributed to this carton. |
| 10 | Operators (3-5 people) | Each operator picks their pouch type and places into carton. Pouch validation need to be done | Reader at each position scans the pouch as it passes. System records pouch serial, pouch type, pick timestamp, operator position. Pick status shown on operator screen: PENDING → CONFIRMED. |
| 11 | System | Carton filling ends — sensor t2 event received | DIGITAL records t2 timestamp. All pouch serials between t1 and t2 attributed to this carton. Fill count validated vs recipe qty. |
|  |  | Step 4 & 5 would be repeated for every operator workstation. |  |
| 12 | System | Fill count validation | If fill count = recipe qty: carton proceeds to check weigher. If mismatch: carton held, operator alerted, supervisor override required. |
| 13 | System | Final check weigher result received for carton. If it is rejected, then conveyor stops diverted to rework queue manually. | PASS: DIGITAL generates permanent Carton ID, triggers QR label print. FG inventory posted. FAIL: carton diverted to rework queue. |
| 14 | Secondary Pack Operator | Labels carton (label printed automatically at CW pass) | Carton QR label contains full genealogy: Carton ID, Batch, Mfg/BBD dates, RM Lot IDs, packed-by station, datetime. |

### 3.7.4 Exception Handling — Secondary Packing

| Exception Code | Trigger Condition | System Action | Required Role | Resolution Options |
| --- | --- | --- | --- | --- |
| EX-SPK-01 | Carton fill count (t1-t2 method) ≠ Recipe qty | Carton release blocked. Alert on supervisor screen and operator displays. | Production Supervisor | Approve Recount Override ¦ Physical Recount ¦ Scrap Carton |
| EX-SPK-02 | Final check weigher FAIL for carton | Carton diverted to rework queue. Alert raised to QA. | QA Supervisor | Rework (re-open and recount) ¦ Scrap |
| EX-SPK-03 | Operator position pick missed (no scan in time window) | Amber alert on that operator's screen. Supervisor notified. | Supervisor | Verify Physical Pick ¦ Override ¦ Halt Line |
| EX-SPK-04 | Assigned tray depleted (out of pouches) | Low tray alert on operator screen. ISA issuance request triggered. | ISA Operator | Issue next tray from ISA |
| EX-SPK-05 | Carton QR label print failure | Alert raised. Carton held as PENDING_LABEL. Printer status shown. | Production Supervisor | Retry Print ¦ Manual Label (override) |

## 3.8 Carton Check Weigher Reject & Mandatory QA Rework

When the final check weigher rejects a carton due to SKU weight mismatch, the carton is diverted to a separate rework table line. The Track and Trace system manages the complete rework lifecycle: identifying missing pouches, creating a rework task for the secondary pack operator, enforcing mandatory QA re-validation, and tracking the reworked carton back through the check weigher before it proceeds to case sealing.

### 3.8.1 Carton Rework Flow — Step by Step

| Step # | Actor | Action Performed | System Response / Outcome |
| --- | --- | --- | --- |
| 1 | System | Check weigher rejects a carton — FAIL event received | DIGITAL records the CW-FAIL against the carton sequence number. Carton status → CW_REJECTED. Carton physically diverted to rework table line. Alert sent to Secondary Pack Supervisor and QA. |
| 2 | Secondary Pack Operator | Physically opens carton, counts actual pouches, and records rework reason | Operator selects rework reason from dropdown (e.g. Missing Pouch, Wrong Pouch Type, Damaged Pouch, Packaging Defect). System displays expected vs. actual pouch count per recipe line. |
| 3 | System | Rework task created and assigned | System creates a Rework Task (CRWK-[sequence]-NNN) linked to the original carton sequence. Task assigned to secondary pack operator at rework station. Task visible on supervisor screen with priority flag. |
| 4 | Secondary Pack Operator | Fixes the carton — adds missing pouches, removes excess/damaged pouches | Each corrective action (pouch added or removed) is scanned and recorded. System validates each added pouch against the recipe (correct type, from approved tray). Corrected genealogy updated. |
| 5 | QA Inspector | MANDATORY: QA Inspector performs physical validation of reworked carton before it enters the check weigher | QA Inspection task auto-created on QA screen. QA Inspector scans carton, records physical inspection result (OK / NOT OK) per carton. If NOT OK: carton sent back to rework. If OK: carton cleared for CW pass. |
| 6 | Secondary Pack Operator | Closes carton and places it before the check weigher again for re-weighing | System flags carton as REWORK_PENDING_CW. The carton retains its original carton sequence number — it is not reassigned a new ID at this stage. |
| 7 | System | Reworked carton passes through check weigher | System identifies the carton as a rework carton (REWORK flag in carton record). CW result recorded. If PASS: carton status → CW_PASS_REWORK. Proceeds to case sealing. If FAIL again: exception raised, Production Manager decision required (scrap or second rework attempt). |
| 8 | System | Reworked carton proceeds to case sealing and serial number printing. | Carton genealogy updated with rework history. Final Carton ID generated. QR label printed with REWORK flag visible in the audit trail (not on the consumer-facing label). FG inventory posted. |

### 3.8.2 Exception Handling — Carton Rework

| Exception Code | Trigger Condition | System Action | Required Role | Resolution Options |
| --- | --- | --- | --- | --- |
| EX-CRW-01 | CW-rejected carton not presented at rework table within configured time window (e.g. 15 min) | Alert to supervisor. Carton status = MISSING_FROM_REWORK. | Production Supervisor | Locate carton ¦ Mark as Scrapped |
| EX-CRW-02 | Reworked carton fails check weigher a second time | Production Manager escalation required. Carton blocked from further automatic rework. | Production Manager | Authorise Second Rework ¦ Scrap Carton |
| EX-CRW-03 | QA mandatory inspection not completed within shift | Alert to QA Supervisor and Production Manager. Reworked carton held. | QA Supervisor | Complete Inspection ¦ Escalate |
| EX-CRW-04 | QA inspection result = NOT OK | Carton returned to rework queue. Second rework attempt counter incremented. Manager notified if second failure. | Production Manager | Approve Second Rework ¦ Scrap |
| EX-CRW-05 | Pouch added during rework scan fails validation (wrong type / already used) | Scan rejected. Error shown. Operator cannot add invalid pouch. | Secondary Pack Operator | Use correct pouch from approved tray |

## 3.9 Carton Case Closure & Serial Number Printing

After a carton passes the final check weigher (first pass or post-rework), it proceeds to case sealing. The case sealer closes the carton. The carton then passes through the serial number printing machine which prints the permanent QR / barcode label on the outer carton. DIGITAL tracks this transition and updates the carton record to FG status.

### 3.9.1 Step-by-Step Workflow

| Step # | Actor | Action Performed | System Response / Outcome |
| --- | --- | --- | --- |
| 1 | System | Check weigher PASS event received for carton (first pass or rework pass) | DIGITAL marks carton as CW_PASSED. Carton eligible for case sealing. Permanent Carton ID generated (CART-YYYYMMDD-SEQ). Rework flag included in internal record if applicable. |
| 2 | Automated (Case Sealer Machine) | Carton physically passes through case sealer — flaps closed and sealed | Case sealing event recorded from Stellar SQL (if event available). Carton status → CASE_SEALED. Timestamp recorded. |
| 3 | Automated (Serial Number Printer) | Carton passes through serial number printing machine | System sends Carton ID, Product SKU, Batch ID, Manufacturing Date, Best Before Date, Net Weight, Pouch Count, All RM Vendor Lot IDs, and Packing Date/Time to label printer via ZPL. Carton QR label printed on outer carton surface. |
| 4 | System | Good volume count updated at end of order | Running Good Carton Count for the shift and production order incremented. Displayed prominently on Secondary Packing Supervisor screen. Carton status → LABELLED. Available for palletising. |
| 5 | Secondary Pack Operator | Confirms case closure and label print (if manual confirmation is required by plant) | Optional: operator scans carton QR label to confirm label is correctly applied. System verifies scan matches expected Carton ID. If mismatch: alert raised. |

## 3.10 Carton Palletising & Pallet Formation

After labelling, each good carton is placed on a designated pallet by the palletiser (manual or automated). Each pallet is assigned a unique Pallet ID. A strict single-SKU rule is enforced — no mixing of different products on the same pallet. When a pallet is complete (or on operator instruction), DIGITAL prints a QA Inspection Required label and notifies the QA person for pallet inspection before the pallet moves to FG Warehouse.

### 3.10.1 Step-by-Step Workflow

| Step # | Actor | Action Performed | System Response / Outcome |
| --- | --- | --- | --- |
| 1 | Palletiser Operator | Begins a new pallet — scans or creates Pallet ID on the palletising station screen | System creates Pallet ID (PLT-YYYYMMDD-SEQ). Operator selects Product SKU for this pallet. Single-SKU rule enforced: system blocks adding a different SKU to a pallet that already has cartons of another SKU. |
| 2 | Palletiser Operator | Cartons are placed on the pallet (manual stacking or automated palletiser) | Each carton scanned as it is placed on the pallet. System records: Carton ID → Pallet ID link. Running carton count on pallet displayed. Target cartons per pallet layer/total shown (from Pallet Definition master per SKU). |
| 3 | System | Pallet reaches defined carton count (or operator declares pallet full) | Pallet status → COMPLETE. System auto-prints QA Inspection Required label on printer at the palletising station. QA Inspector receives notification: 'Pallet [PLT-ID] ready for inspection — [SKU] [Carton Count] cartons.' |

### 3.10.2 Exception Handling — Palletising

| Exception Code | Trigger Condition | System Action | Required Role | Resolution Options |
| --- | --- | --- | --- | --- |
| EX-PLT-01 | Operator attempts to scan a carton of different SKU onto an active pallet | Scan blocked. Error: 'Pallet [ID] is locked to [SKU-A] — cannot add [SKU-B]. Start a new pallet for [SKU-B].' | Palletiser Operator | Scan correct SKU carton ¦ Start new pallet |
| EX-PLT-02 | Carton scanned on pallet has status ≠ LABELLED (e.g. CW_REJECTED or REWORK_PENDING) | Scan rejected. Error: 'Carton [ID] is not cleared for palletising — current status: [STATUS].' | Production Supervisor | Resolve carton status ¦ Remove from pallet area |
| EX-PLT-03 | Carton already attributed to another pallet | Scan rejected. Duplicate pallet assignment blocked. | Production Supervisor | Investigate ¦ Correct pallet assignment |
| EX-PLT-04 | Pallet carton count at close ≠ Pallet Definition target | Amber warning. Supervisor acknowledgement required to close short/over-count pallet. | Production Supervisor | Acknowledge with reason ¦ Add/remove cartons |

| OQ-06 | OPEN QUESTION: Please provide the pallet definition details per SKU: cartons per pallet, stacking layers, pallet type (CHEP/Euro/Own). This is required to configure the Pallet Definition master data before UAT. |
| --- | --- |

## 3.11 QA Inspection of Pallets

After a pallet is formed and the QA Inspection Required label is printed, a QA Operator performs random carton inspection on the pallet. Each carton on the pallet can be individually accepted or rejected. Rejected cartons are removed and returned to the secondary packing rework queue. The remaining accepted cartons on the pallet are released to the FG Warehouse.

### 3.11.1 Step-by-Step Workflow

| Step # | Actor | Action Performed | System Response / Outcome |
| --- | --- | --- | --- |
| 1 | QA Inspector | Receives pallet inspection notification (screen alert) | QA screen shows: Pallet ID, SKU, Carton Count, Formation Timestamp. Inspection timer starts. |
| 2 | QA Inspector | Scans Pallet ID QR label to begin inspection | System displays all carton IDs on the pallet. QA Inspector selects the cartons to inspect (random sampling or 100% — configurable by QA Supervisor per SKU/shift). |
| 3 | QA Inspector | For each inspected carton: scans carton, records OK or NOT OK | OK: carton status confirmed as QA_PASSED. NOT OK: carton flagged QA_FAILED with mandatory reason code entry. |
| 4 | QA Inspector | For QA_FAILED cartons: removes them physically from the pallet | Operator removes carton and scans it as removed. System updates pallet: carton removed, carton count decremented. QA_FAILED carton enters secondary packing rework queue (linked to original carton ID). |
| 5 | QA Inspector | Completes pallet inspection — closes inspection | System records inspection summary: Total Inspected, Passed, Failed, Removed. Pallet status updated: RELEASED_TO_FG (if any cartons remain) or FULLY_REJECTED (if all cartons failed). |
| 6 | System | Pallet released to FG Warehouse | Pallet status → FG_WAREHOUSE_PENDING. FG inventory records updated with pallet contents. Warehouse operator notified of incoming pallet. |

### 3.11.2 Exception Handling — QA Pallet Inspection

| Exception Code | Trigger Condition | System Action | Required Role | Resolution Options |
| --- | --- | --- | --- | --- |
| EX-QAP-01 | QA inspection not initiated within configured time of pallet completion (e.g. 30 min) | Alert escalated to QA Supervisor and Production Manager. Pallet held. | QA Supervisor | Assign Inspector ¦ Acknowledge Delay |
| EX-QAP-02 | All cartons on pallet fail QA inspection | Pallet status = FULLY_REJECTED. Production Manager escalation. Entire pallet returned to rework. | Production Manager | Authorise Full Rework ¦ Scrap Pallet |
| EX-QAP-03 | QA Inspector tries to release pallet without inspecting minimum required cartons (in RANDOM mode) | System blocks pallet release. Error: 'Minimum [N] cartons must be inspected for this pallet — [X] inspected so far.' | QA Inspector | Complete minimum inspections |

## 3.12 Finished Goods Warehouse Receipt

After QA pallet inspection is complete, the pallet is moved to the FG Warehouse. The Warehouse Person acknowledges receipt by scanning all cartons on the handheld device, recording the aisle/bay storage location, and updating the FG inventory. This section defines the Track and Trace monitoring scope for FG Warehouse — it is important to note the scope boundary: FG Warehouse putaway and pick operations (SAP WM/EWM) are out of scope for Track and Trace. Track and Trace monitors the inbound receipt event and flags any damaged carton issues.

SCOPE BOUNDARY — FG Warehouse

IN SCOPE (DIGITAL): Inbound FG receipt acknowledgement, carton scanning at warehouse gate, aisle-bay location capture, damaged carton flagging, timestamp recording.

OUT OF SCOPE (SAP): FG putaway logic, pick orders, delivery creation, goods issue, dispatch processing — all managed in SAP WM/EWM as a separate system.

DIGITAL provides the FG receipt record as a read-only reference; it does not replicate SAP WM operations.

### 3.12.1 Step-by-Step Workflow

| Step # | Actor | Action Performed | System Response / Outcome |
| --- | --- | --- | --- |
| 1 | Warehouse Person | Receives pallet at FG Warehouse gate — system notification on handheld | Track and Trace sends inbound notification: Pallet ID, SKU, Carton Count, source (Secondary Packing line). Warehouse Person opens receipt screen on handheld. |
| 2 | Warehouse Person | Scans Pallet ID QR label to confirm receipt | System displays expected carton list for this pallet. Carton count confirmation required. |
| 3 | Warehouse Person | Scans each carton QR label on the handheld | Each scan confirmed against pallet manifest. Running count shown: X of N cartons scanned. Discrepancies (extra or missing carton) flagged immediately. |
| 4 | Warehouse Person | Scans fixed aisle/bay location label at the storage position | System records Pallet ID → Aisle-Bay location mapping. FG inventory updated with location. Timestamp recorded. |
| 5 | Warehouse Person | Flags any damaged cartons identified during material handling | Damaged carton scan → system prompts: 'Mark as DAMAGED — send for rework?' Operator confirms. Damaged carton removed from FG inventory. Rework request sent back to secondary packing shopfloor. |
| 6 | System | FG inventory updated | Pallet and all constituent cartons reflected in DIGITAL FG inventory: Pallet ID, Carton IDs, SKU, Batch ID, Location, Receipt Timestamp, QA clearance status. Available for SAP downstream reference. |

### 3.12.2 Exception Handling — FG Warehouse Receipt

| Exception Code | Trigger Condition | System Action | Required Role | Resolution Options |
| --- | --- | --- | --- | --- |
| EX-FGW-01 | Carton count at receipt < pallet manifest count | Alert: '[N] cartons missing from pallet manifest'. Warehouse Supervisor notified. | Warehouse Supervisor | Locate missing cartons ¦ Acknowledge shortage with reason |
| EX-FGW-02 | Carton scanned at warehouse not on any pallet manifest | Alert: 'Carton [ID] not found on any active pallet manifest'. Possible label print error or misrouted carton. | Production Supervisor | Investigate origin ¦ Assign to correct pallet or quarantine |
| EX-FGW-03 | Damaged carton identified — rework request sent | Rework request created in DIGITAL. Secondary packing supervisor notified. Carton removed from FG inventory. | Secondary Pack Supervisor | Assign rework task ¦ Track rework completion |
| EX-FGW-04 | Warehouse receipt not acknowledged within configured window after pallet dispatch | Alert to Warehouse Supervisor and Production Manager. Pallet status = IN_TRANSIT_UNCONFIRMED. | Warehouse Supervisor | Acknowledge receipt ¦ Investigate pallet location |

DESIGN RECOMMENDATION: FG Warehouse — SAP Integration Design Note

While FG Warehouse putaway and pick are handled in SAP, Carbynetech recommends that DIGITAL generates a structured FG Receipt record in a format compatible with SAP Goods Receipt (Movement Type 501/101) for future integration.

Key fields to align: Material Code (SAP format), Batch Number (map to DIGITAL Batch ID), Plant/Storage Location (map to Aisle-Bay), Quantity, Posting Date.

This ensures a Phase 2 SAP integration does not require data restructuring — the mapping is done at design time in v1.

## 3.13 Liquid Receipt / GRN

| Step # | Actor | Action Performed | System Response / Outcome |
| --- | --- | --- | --- |
| 1 | Gate Guard | Records truck arrival number at gate | System logs truck arrival event with timestamp |
| 2 | Liquid Store Operator | Opens 'New GRN' screen on IPC or Android App | System displays PO selection screen with list of open POs |
| 3 | Liquid Store Operator | Selects or searches Purchase Order / Transfer Order | System auto-populates: Vendor Name, Material List, Expected Quantities |
| 4 | Liquid Store Operator | Scans vendor barcode or manually enters liquid material details | If GS1 barcode: auto-populate Vendor Lot, Material Code, Qty, Expiry. If manual: all fields editable. |
| 5 | Liquid Store Operator | Enters: Vendor Lot No., DC No., Received Qty, Pallet/Container Count, Vehicle No. | System validates mandatory fields. Checks quantity vs PO (tolerance configurable ±3%). If variance > tolerance, GRN flagged for approval. |
| 6 | Liquid Store Operator | Submits GRN | System generates MES Internal Lot ID (format: LOT-YYYYMMDD-NNN). Sets material status = UNDER_INSPECTION. Auto-triggers IQC request. Prints GRN label. |
| 7 | Store Operator | Prints pallet labels and places material in quarantine zone | Label includes: Lot ID, Material, Qty, Vendor, Expiry, QC: PENDING (barcode/QR). Dock-to-store timer starts. |
| 8 | QA Supervisor | Receives pending inspection alert on QA station screen | Alert shows: Material, Vendor, Lot ID, Qty, GRN time, Waiting duration. QC pending timer visible. |
| 9 | QA Supervisor | Records inspection decision: PASS / HOLD / REJECT | PASS: material → APPROVED, green label printed. HOLD: material blocked, yellow label, re-inspection date set. REJECT: inventory zeroed, red label, return-to-vendor initiated. |
| 10 | Store Operator | Scans Aisle-Bay location label and confirms put away. No of pallets and no of Container on that pallet, needs to enter and that info on barcode for trace purpose. Pallet sticker would have Liquid RM SKU, lot Id, Pallet Id & Container count. Mfg date. | System records Lot ID → Location mapping. Inventory updated. Material available for production issuance (if APPROVED). |

## 3.14 Dispatch Order

## Create a dispatch order, select New Order, and input the FG, Qty, Sales Order, SAP Delivery, Pickup Location, Delivery destination(SAP Plant), and Item details. Finally, assign a driver, transporter, Vehicle Number and click Submit to process the shipment.

| Step # | Actor | Action Performed | System Response / Outcome |
| --- | --- | --- | --- |
| 1 | Dispatch Operator | Creates a new Dispatch Order by entering Finished Good (FG), Quantity, Sales Order, SAP Delivery Number, Pickup Location, Delivery Destination (SAP Plant), Item Details, Driver, Transporter, and Vehicle Number. | System validates the details and creates the Dispatch Order. |
| 2 | Warehouse Operator | Scans the cartons and liquid containers against the Dispatch Order during loading. | System validates the scanned items against the dispatch quantity and records the loaded inventory. |
| 3 | Dispatch Operator | Reviews the dispatch information and submits the shipment. | System confirms shipment dispatch and generates the dispatch confirmation. |
| 4 | System | Completes the dispatch transaction. | Inventory is updated, stock is deducted, and shipment details are available for tracking and reporting. |

# 4. Traceability & Genealogy

The genealogy engine maintains a directed graph of production entities from vendor lot to finished goods carton.

| Level | From Entity | To Entity | Link Created By |
| --- | --- | --- | --- |
| 1 | Vendor Lot | MES Internal Lot ID | Store Operator (GRN) |
| 2 | MES Internal Lot | QA Inspection Record | QA Supervisor |
| 3 | MES Lots (multiple) | Batch ID | Dumping Operator (bag scans) |
| 4 | Batch ID | Pouch Serials | System (Stellar Rovema events) |
| 4a | Batch ID | Rework Order ID | QA Supervisor (rework inspection) |
| 5 | Pouch Serial | Tray ID | System (RFID + tray assignment) |
| 6 | Tray ID | Carton ID (LABELLED — post case seal) | System (on CW PASS + case seal + QR label print) |
| 7 | Carton ID/Container ID | Pallet ID | Palletiser Operator (carton scan at palletising station) |
| 8 | Pallet ID | Dispatch Order |  |

# 5. Roles & Permissions Matrix

| Module / Function | Store Opr+WareHouse Opr | QA Supvr | Dump Opr | Prim Pack | Sec Pack | Prod Mgr/Planner | Mgmt | Admin |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Create GRN | ✓ RW | — | — | — | — | View | — | — |
| Record IQC Decision | — | ✓ RW | — | — | — | View | — | — |
| Issue Material | ✓ RW | — | — | — | — | View | — | — |
| Material Requisition (Dumping) | — | — | ✓ RW | — | — | View | — | — |
| Dumping Scan & Confirm | — | — | ✓ RW | — | — | View | — | — |
| Rework Order — QA Decision | — | ✓ RW | — | — | — | View | — | — |
| Primary Packing Operations | — | — | — | ✓ RW | — | View | — | — |
| ISA Tray Receipt & Issuance | — | — | — | ✓ RW | ✓ RW | View | — | — |
| Carton Rework Task Management | — | — | — | — | ✓ RW | View | — | — |
| QA — Reworked Carton Inspection | — | ✓ RW | — | — | — | View | — | — |
| Palletising Operations | — | — | — | — | ✓ RW | View | — | — |
| QA Pallet Inspection | — | ✓ RW | — | — | — | View | — | — |
| FG Warehouse Receipt | — | — | — | — | — | View | — | — |
| Approve Override (Exceptions) | — | — | — | — | — | ✓ RW | — | — |
| Release Production Orders | — | — | — | — | — | ✓ RW | — | — |
| Material Balance Sign-off | — | — | — | — | — | ✓ RW | — | — |
| View Plant Dashboard | — | View | — | — | — | ✓ RW | ✓ RW | — |
| Traceability Search | View | ✓ RW | — | — | — | ✓ RW | ✓ RW | ✓ RW |
| Material & Recipe Master | — | — | — | — | — | View | — | ✓ RW |
| User & Role Management | — | — | — | — | — | — | — | ✓ RW |

# 6. Hardware Sizing Recommendation

Hardware procurement is outside Carbynetech scope. The following is a minimum specification guide for RCPL planning. These costs are NOT included in the software investment.

## 6.1 DIGITAL Application Server (Recommended Minimum)

The below sizing is for the Production Server only. An additional replica server will be required for Development and Testing purposes.

| Component | Specification |
| --- | --- |
| CPU | Intel Xeon E-2300 series— 8 cores / 16 threads, 12–16 vCPU ≥ 3.0 GHz |
| RAM | 128 GB DDR4 ECC |
| Primary Storage | 500 GB NVMe SSD (RAID-1 recommended for OS + App) |
| Data Storage | 2 TB SATA SSD or 4 TB HDD (RAID-5/RAID-6 for App DB + Logs) |
| Network | 2× 1 Gbps Ethernet (bonded); connectivity to plant LAN/OT VLAN |
| OS | Windows Server 2022 Standard or Ubuntu 22.04 LTS Server |
| UPS / Power | Online UPS, 1500 VA minimum, 30-min backup recommended |
| Form Factor | 1U/2U Rack-mount or Tower (as per plant infrastructure) |

## 6.2 Unified SQL Event Server (Stellar-owned, for reference)

The Unified SQL Server where Siemens events are written by Stellar should be accessible to the DIGITAL server on the plant network. Carbynetech requires read-only SQL access. Minimum: 32 GB RAM, 8-core CPU, 1 TB SSD, MS SQL Server 2019/2022 Standard. Licensing is Customer's responsibility.

## 6.3 IPC Station Screens

| Spec | Requirement |
| --- | --- |
| Processor | Intel Core i5 (10th Gen+) or equivalent Industrial IPC |
| RAM | 8 GB RAM minimum |
| Display | 21" or larger, 1920×1080, touch-enabled preferred |
| OS & Browser | Windows 10/11 IoT or standard; Chrome / Edge Chromium |
| Network | Wired LAN to plant network |
| Quantity (est.) | 5–6 IPC stations across: Receiving, Dumping, Primary Packing ×2, Secondary Packing, QA |

## 6.4 Android Scanner Devices

| Spec | Requirement |
| --- | --- |
| Device Type | Industrial Android Handheld (e.g., Zebra TC21, Honeywell CT47, or equivalent) |
| Android Version | Android 10 or above |
| Scanner Type | 2D imager (QR + 1D barcode), IP54 or higher rating |
| Connectivity | Wi-Fi 802.11 a/b/g/n/ac; plant-wide Wi-Fi coverage required |
| Quantity (est.) | 4–6 devices (1 per station + 1–2 spares) |

## 6.2 IPC Station Screens

| Station | Count | Display Size | Notes |
| --- | --- | --- | --- |
| RM Receiving | 1 | 21"+, 1920×1080, touch | GRN creation and store operations |
| QA Inspection | 1 | 21"+, 1920×1080 | IQC decisions and rework orders |
| Dumping Material Requisition | 1 | 21"+, 1920×1080 | Material requisition to store |
| Dumping / Batching | 3 | 21"+, 1920×1080 | Bag scan operations |
| Primary Packing | 4 | 21"+, 1920×1080 | Machine monitor + tray assignment (one per machine) |
| ISA | 1 | 21"+, 1920×1080 | Tray receive, locate, issue |
| Secondary Packing Supervisor | 1 | 21"+, 1920×1080 | Order management and exception handling |
| Secondary Packing Operator Positions | 3-5 | 19"+, 1920×1080 | Operator pick display per position |

# 10. Glossary

| Term | Definition |
| --- | --- |
| BOM | Bill of Materials — raw materials and quantities required to produce one batch |
| Carton ID | Unique identifier for a finished goods carton, generated at secondary packing after CW pass |
| DIGITAL | The Track & Trace Digital Application Suite delivered by Carbynetech |
| FEFO | First Expired, First Out — inventory issuance prioritising material closest to expiry |
| FG | Finished Goods — completed, labelled cartons ready for dispatch |
| GRN | Goods Receipt Note — record of incoming materials from a vendor or inter-plant transfer |
| Giveaway | Weight given above target fill weight for passing pouches — a cost KPI for packing lines |
| ISA | Intermediate Storage Area — buffer zone holding sealed trays between Primary and Secondary Packing |
| IQC | Incoming Quality Control — QA inspection on received raw materials |
| Lot ID | MES Internal Lot Identifier generated at GRN — primary traceability anchor for incoming material |
| MES | Manufacturing Execution System — digital system managing production floor operations |
| OT Layer | Operational Technology layer — PLCs, SCADA, sensors managed by Stellar |
| Pouch Serial | Unique serial number assigned to each filled pouch by the Rovema machine |
| RCPL | Reliance Consumer Products Limited — the client |
| RFID | Radio Frequency Identification — technology used to tag and track plastic trays |
| RM | Raw Material — ingredients used in beverage production |
| Rovema | The pouch filling machine brand used at Primary Packing |
| Rework Order | A sub-order linked to the parent job order, created to track check-weigher rejected pouches through QA inspection and re-processing |
| Staging Location | One of 3 (TBC) physical positions at the dumping floor where RM bags are held prior to batch scanning |
| Stellar | The automation partner delivering the OT layer, SCADA, and SQL event database |
| t1 / t2 | Sensor timestamps: t1 = carton filling start, t2 = carton filling end. Determines which pouches are attributed to a carton. |
| Tray | Reusable plastic container holding N pouches; tagged with RFID |
| WIP | Work In Progress — intermediate material or semi-finished product |
| ZPL | Zebra Programming Language — command language for Zebra label printers |

# 11. Document Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared By — Carbynetech |  |  | June 2026 |
| Reviewed By — RCPL (Factory Team) |  |  |  |
| Approved By — RCPL (Project Sponsor) |  |  |  |

© 2025 Carbynetech Technologies Pvt. Ltd., Hyderabad  |  CONFIDENTIAL — Restricted Distribution